# Configuration Reference

All configuration lives in the config/ folder as plain JSON files:

| File | Purpose | Edit by hand? |
| --- | --- | --- |
| config.template.json | Master template with your station defaults. Copied to config.json on first run. | Yes (before first run) |
| config.json | The live configuration the app actually uses. Auto-created from the template. | Yes (after first run) |
| themes.json | Saved color palettes and the active theme. | Usually via the UI |
| users.json | User accounts (passwords are bcrypt-hashed). | Managed via the UI |

After editing config.json, restart the server for hardware/connection changes (serial, UDP, MQTT, FlexRadio) to take effect.

## site — Web server & security

"site": {
  "site_name": "WB0Z - Flex 8600",
  "station_callsign": "WB0Z",
  "http_port": 3000,
  "bind_address": "0.0.0.0",
  "jwt_secret": "CHANGE_ME_TO_A_LONG_RANDOM_STRING",
  "jwt_expiry": "12h"
}

| Key | Meaning |
| --- | --- |
| site_name | Title shown in the top bar and login page. |
| station_callsign | Your callsign, shown in the header. |
| http_port | TCP port the web UI/API listen on. Change if 3000 is taken. |
| bind_address | 0.0.0.0 = reachable on the whole LAN; 127.0.0.1 = local only. |
| jwt_secret | Secret used to sign login tokens. Leave the CHANGE_ME… placeholder and the app will auto-generate a strong random secret on first run. |
| jwt_expiry | How long a login lasts (12h, 7d, etc.). |

## flexradio — FlexRadio 8600 (SmartSDR)

"flexradio": {
  "enabled": true,
  "host_mode": "automatic",
  "host": "",
  "discovery_port": 4992,
  "staleness_timeout_ms": 15000,
  "watchdog_interval_sec": 5,
  "subscriptions": [ ... ],
  "amp_integration": { "amp_on_rf_power": 30, "amp_off_rf_power": 90, "max_rf_power_with_amp": 50 },
  "meters": { ... }
}

| Key | Meaning |
| --- | --- |
| enabled | Turn the FlexRadio integration on/off. |
| host_mode | automatic = discover the radio on UDP 4992; manual = use host. |
| host | Radio IP address (only used when host_mode is manual). |
| discovery_port | FlexRadio discovery port (default 4992). |
| staleness_timeout_ms | If no data for this long, the link is marked down. |
| watchdog_interval_sec | How often the staleness check runs. |
| subscriptions | SmartSDR subscription commands sent on connect. {Client_ID} is substituted. |
| amp_integration.amp_on_rf_power | RF power (W setting) applied when the amp goes to operate. |
| amp_integration.amp_off_rf_power | RF power applied when the amp returns to standby. |
| amp_integration.max_rf_power_with_amp | Hard ceiling on RF power while the amp is engaged. |
| meters | Map of meter definitions (topic match + display range). Used to scale the gauges. Each meter has min/max and optionally conversion (dbm_to_watts) or global_var. |

## serial — Palstar LA-1K amplifier

"serial": {
  "palstar_la1k_amp": {
    "enabled": true,
    "serial_port": "COM7",
    "baud_rate": 9600,
    "data_bits": 8,
    "parity": "none",
    "stop_bits": 1,
    "response_timeout_ms": 100,
    "protocol": { ... }
  }
}

| Key | Meaning |
| --- | --- |
| enabled | Enable serial control of the amp. |
| serial_port | Windows COM port (e.g. COM7). Check Device Manager → Ports. |
| baud_rate, data_bits, parity, stop_bits | Serial line settings (9600/8/N/1). |
| response_timeout_ms | Per-poll response timeout. |
| protocol.poll_command | Status poll string (AR1;). |
| protocol.poll_interval_sec | How often to poll (seconds). |
| protocol.operate_command / standby_command | Commands for operate (AM1;) / standby (AM2;). |
| protocol.antenna_select_command | Antenna command; {n} is replaced by 1/2/3. |
| protocol.frequency_command | Frequency command; {freq_padded_8} is an 8-digit zero-padded kHz value. |
| protocol.band_codes | Maps the amp’s band code field to a label. |
| protocol.key_status_codes | Maps the amp’s key-status field to a label. |

If serialport cannot load or the port is missing, the amp runs in simulation mode and the rest of the app keeps working.

## udp — PST Rotator & Palstar HF-Auto Tuner

### PST Rotator

"pst_rotator": {
  "enabled": true,
  "listen_port": 12001,
  "send_address": "127.0.0.1",
  "send_port": 12000,
  "poll_command": "<PST>AZ?</PST>",
  "poll_interval_sec": 10,
  "set_azimuth_command": "<PST><AZIMUTH>{degrees}</AZIMUTH></PST>",
  "stop_command": "<PST><STOP>1</STOP></PST>",
  "presets": [ { "label": "Caribbean", "value": 100 }, ... ]
}

| Key | Meaning |
| --- | --- |
| listen_port | UDP port the app listens on for azimuth data from PstRotator (12001). |
| send_address / send_port | Where commands are sent (PstRotator on 127.0.0.1:12000). |
| poll_command / poll_interval_sec | Azimuth poll string and interval. |
| set_azimuth_command | Command to turn the rotator; {degrees} is replaced. |
| stop_command | Emergency stop command. |
| presets | Heading shortcuts shown in the dashboard dropdown. |

### Palstar HF-Auto Tuner

"palstar_hf_auto_tuner": {
  "enabled": true,
  "listen_port": 13080,
  "send_address": "127.0.0.1",
  "send_port": 12020,
  "broadcast": true,
  "watchdog_timeout_sec": 20,
  "commands": { "select_antenna_1": "<?xml ...>", "mode_auto": "<?xml ...>", ... },
  "antenna_rules": { "1": {"name":"HexBeam","force_mode":"bypass"}, ... }
}

| Key | Meaning |
| --- | --- |
| listen_port | UDP port for incoming XML status (13080). |
| send_address / send_port | Where XML commands are sent (12020). |
| broadcast | Send as a UDP broadcast (matches Node-RED’s broadcast mode). |
| watchdog_timeout_sec | If no data within this time, the tuner is marked offline. |
| commands | Raw XML command strings for antenna/mode selection. |
| antenna_rules | Friendly name per antenna and the tuner mode automatically forced when it is selected (resonant → bypass, non-resonant → auto). |

## mqtt — Home Assistant Mosquitto bridge

"mqtt": {
  "enabled": true,
  "broker": "192.168.1.54",
  "port": 1883,
  "username": "",
  "password": "",
  "client_id": "hamcontrol_web",
  "birth_topic": "hamcontrol/status",
  "will_topic": "hamcontrol/status",
  "publish_interval_sec": 60,
  "topic_prefix": "hamcontrol/global"
}

| Key | Meaning |
| --- | --- |
| enabled | Turn the MQTT bridge on/off. |
| broker / port | Mosquitto broker address (your HA at 192.168.1.54:1883). |
| username / password | MQTT credentials (leave blank if anonymous). |
| client_id | MQTT client identifier. |
| birth_topic / will_topic | Online/offline status topics (retained). |
| publish_interval_sec | How often global state is published. |
| topic_prefix | Topics published as <prefix>/<key> (powerSupply, radioPower, ampPower, ampStatus, amp, tuner, TXStatus). |

## home_assistant — Smart plugs, relays & power control

"home_assistant": {
  "enabled": true,
  "base_url": "http://192.168.1.54:8123",
  "token": "",
  "timeout_ms": 5000,
  "polling_interval_sec": 15,
  "confirm_delay_ms": 600,
  "entities": {
    "power_supply": { "entity_id": "switch.smart_plug",  "state_topic": "" },
    "amplifier":    { "entity_id": "switch.smart_plug_2", "state_topic": "" },
    "radio_relay":  { "entity_id": "switch.shelly1g4_a085e3c0f2c0", "state_topic": "" }
  }
}

| Key | Meaning |
| --- | --- |
| enabled | Turn Home Assistant control on/off. |
| base_url | HA URL, e.g. http://192.168.1.54:8123. |
| token | Home Assistant long-lived access token (Profile → Long-Lived Access Tokens). Required for power control. |
| timeout_ms | HTTP request timeout for HA service calls and state reads. |
| polling_interval_sec | How often the server re-reads device states from HA to stay in sync (set 0 to disable). |
| confirm_delay_ms | Wait after a switch command before reading the state back to confirm it. |
| entities.*.entity_id | The HA entity switched by each power button. |
| entities.*.state_topic | (optional) An MQTT topic that publishes this device’s on/off state for real-time updates. Leave blank to rely on REST polling. |

How power control works: When you toggle a power button, the server calls the Home Assistant REST API — POST {base_url}/api/services/switch/turn_on (or turn_off) with { "entity_id": "<configured id>" } and a Bearer <token> header. This is exactly what the original Node-RED flow did. The MQTT topics under topic_prefix are only status mirrors for HA dashboards; they do not switch the devices. All HA calls are logged with [ha] / [power] prefixes.

State synchronization (UI always reflects reality):

- On startup the server reads the current state of all three devices from HA (GET /api/states/<entity_id>) and broadcasts them to every browser, so the UI shows the actual on/off state — even if devices were already on.

- After every toggle the server re-reads the entity and broadcasts the confirmed state (not just the requested value). The toggle button shows a brief Syncing… state until HA confirms.

- Continuously the server polls HA every polling_interval_sec and, if any state_topic is set, subscribes to those MQTT topics for instant updates. Any change made outside the app (HA app, physical switch) is detected and pushed to all connected clients over WebSocket.

- The Power card shows an HA SYNCED / SYNCING… / HA ERROR / HA OFF badge and the time of the last successful sync. Discrepancies between the UI and the real device state are logged as [ha] STATE CHANGE ….

Set base_url and the token (and confirm the entity_ids) on the Settings screen. The token is write-only in the UI — it is never sent back to the browser; leave the field blank to keep the saved token.

## automation — Behavior toggles

| Key | Meaning |
| --- | --- |
| amp_rf_power_limiting.enabled | Apply the FlexRadio RF power rules when the amp mode changes. |
| tuner_antenna_rules.enabled | Auto-force tuner mode when an antenna is selected. |
| desk_light_tx_rx.enabled | TX/RX desk-light color automation (requires HA). |

## themes.json

{ "active": "dark-amber", "themes": { "dark-amber": { "name": "...", "colors": { "primary": "#ffb000", ... } } } }

- active — id of the currently applied theme.

- themes — map of theme id → { name, colors }. Each colors object maps a CSS variable name (without the leading --) to a hex color.

- Best managed from the Appearance tab in the UI, which writes this file.

## users.json

{ "users": [ { "username": "wb0z", "displayName": "WB0Z", "role": "admin", "passwordHash": "$2a$...", "disabled": false } ] }

- Passwords are stored only as bcrypt hashes — never plain text.

- role is one of admin, normal, viewonly.

- Manage accounts from the Users tab (admin only). To fully reset, delete this file and restart — the default wb0z admin is recreated.